#### Cessna-172S N910CP

# Preflight Cabin

- AIF Front Cover………..Inspect for Airworthiness

- Pitot Tube Cover ..	Remove. Check for blockage.

- POH	Available in the Airplane.

- Weight & Balance	Checked.

- Parking Brake	Set.

- Control Wheel Lock	Remove.

- Ignition Switch	Off.

- Avionics Master Switch	Off.

WARNING

When the master switch is on, using an external power source, or manually rotating the propeller, treat the propeller as if the magnetos switch were on. Do not stand, nor allow anyone else to stand, within the arc of the propeller since a loose or broken wire, or a component malfunction could cause the engine to start.

- Master Switch	On.

10.  Fuel Quantity Indicators	Check Quantity and Ensure Low Fuel Annunciators (L Low Fuel R) are extinguished.

11. Avionics Master Switch	On.

12. Avionics Cooling Fan	..Check Audibly for Operation.

13. Avionics Master Switch	Off.

14. Static Pressure Alternate Source Valve	Off.

15.  Annunciator Panel Switch..	Place and Hold in TST Position and ensure all amber and red anunciators illuminate.

16. Annunciator Panel Test Switch…..	Release. Check that appropriate annunciators remain on.

Note

When Master Switch is turned ON, some annunciators will flash for approximately 10 seconds before illuminating steadily.  When panel TST switch is toggled up and held in position, all remaining lights will flash until the switch is released.

- Fuel Selector Valve	Both.

- Fuel Shutoff Valve.	On (Push Full In).

- Flaps	Extend.

- Pitot Heat…	On. (Carefully check that pitot tube is warm to the touch within 30 seconds.)

- Pitot Heat	Off.

- Master Switch	Off.

- Baggage Door	..Check, lock with key.

Preflight Empennage

- Rudder Gust Lock (if installed)……	Remove.

- Tail Tie-Down	Disconnect.

- Control Surfaces..	.Check freedom of movement and security.

- Trim Tab	Check security.

- Antennas	Check for security of attachment and general condition.

Preflight Right Wing Trailing Edge

- Aileron	..Check freedom of movement and security.

- Flap	Check security & condition.

# Preflight Right Wing

- Wing Tie Down	Disconnect.

- Main Wheel Tire	Check for proper inflation and general condition.

- Fuel Tank Sump Drain Valves	…Drain at least a cupful of fuel from each sump location to check for water, sediment, and proper fuel grade before each flight and after each refueling. If water is observed, take further samples until clear and then gently rock wings and lower tail to the ground to move additional contaminants to the sampling point. Take repeated samples from all fuel drain points until all contamination has been removed. If contaminants are still present, refer to Warning below and do not fly airplane.

WARNING

If, after repeated sampling, evidence of contamination still exists, the airplane should not be flown. Tanks should be drained and system purged by qualified maintenance personnel. All evidence of contamination must be removed before further flight.

4. Fuel Quantity	Check Visually.

5. Fuel Filler Cap	Secure and Vent Unobstructed.

Preflight Nose

- Fuel Strainer Valve	Drain.

- Reservoir Quick Drain & Fuel Selector Quick Drain Valve	Drain at least a cupful from valve.

- Engine Oil Dipstick/Filler Cap		Check oil level and secure. (5 qt min., 8 qt for extended flights)

- Engine Cooling Air Inlets	Clear.

- Propeller & Spinner	Check.

- Air Filter	Check.

- Nose Wheel Strut and Tire	Check.

- Left Static Source Opening	Check.

# Preflight Left Wing

- Left Fuel Quantity	Visually Check.

- Fuel Filler Cap	Secure & Vent unobstructed.

- Fuel Tank Quick Drain Valves.	Drain.

- Main Wheel Tire	Check Condition.

# Preflight Left Wing Leading Edge

- Fuel Tank Vent Opening	Check for blockage.

- Stall Warning Opening	Check for blockage.

- Wing Tie Down	Disconnect.

- Landing/Taxi light(s)	Check.

# Preflight Left Wing Trailing Edge

- Left Aileron	Check.

- Left Flap	Check.

##### PASSENGER BRIEF

- Seat Belts / Shoulder Harness

- Personal Electronic Devices off

- Air Vents / Comfort

- Fire Extinguisher Location / Operation

- Emergency Procedures & Exits

##### MISSION BRIEF

- Mission Objective

- Destination, WX, Route, Alt, ETE

- NOTAMS

- Crew Coordination & CRM

- Sterile Cockpit Procedures

- Cockpit Layout

- Intercom & Radio Usage

- Seats, Seatbelts, Doors

- Emergency Action & Equipment

Before Starting Engine

- Preflight Inspection	Complete.

- Passenger Brief	Complete.

- Seats / Belts / Shoulder Harness	Adjust and lock, check initial reel (front & rear).

- Brakes	Test & Set.

- Circuit Breakers	Check In.

- Electrical Equipment, Autopilot	Off.

Caution

The avionics master switch must be off during engine start to prevent possible damage to avionics.

- Avionics Master Switch	Off.

- Fuel Selector Valve	Both.

- Fuel Shutoff Valve	On (push full in).

- Avionics Circuit Breakers.	Check In.

# Starting Engine (Using Battery)

- Throttle Control	Open ¼ Inch.

- Mixture Control	Idle Cut Off.

- Propeller Area	Clear.

- Master Switch	On.

- Flashing Beacon	On.

- Auxiliary Fuel Pump Switch	On.

- Mixture	…...Advance to obtain stable fuel flow (usually 3 to 5 seconds) then return to Idle Cut Off position.

Note

If engine is warm, omit priming procedure of step 7 above.

- Ignition Switch	Start.

- Mixture..	Advance smoothly to Rich when engine starts.

Note

If the engine floods, turn off aux. fuel pump, place mixture in idle cut off, open throttle ½ to full, and crank engine.  When the engine fires, advance mixture to the full rich & retard throttle promptly.

- Oil Pressure	Check.

- Auxiliary Fuel Pump	Off.

- Nav Lights	On as required.

- Avionics Master Switch	On.

- Radios	On.

- Flaps	Retract.

Taxi

- Mixture Control	Lean as required.

- Heat / Vents / Defrost	As Required.

- Brakes	Release and Test.

- Attitude Indicator	Verify Proper Ops.

- Turn Coordinator	Verify Ops.

- H.I. & Compass	Verify Proper Ops.

Before Takeoff - Run-Up

- Parking Brake	Set.

- Passenger Seat Backs…Most upright position.

- Seats and Seat Belts..	Check Secure.

- Cabin Doors	Closed and Locked.

- Flight Controls	Free & Correct.

- Flight Instruments	Check & Set.

- Fuel Quantity	Check.

- Mixture	Rich.

- Fuel Selector Valve	Recheck Both.

- Elevator Trim	Set for Takeoff.

- Throttle Control	1800 RPM.

- Magnetos	Check (RPM drop 150 or 50 differential between magnetos.)

- Suction Gage	Check.

- Engine Instruments	Check.

- Ammeters	Check.

- Annunciator Panel	Ensure no annunciators are illuminated.

- Throttle	Check Idle.

- Throttle	1000 RPM or less.

- Throttle Friction Lock	Adjust.

- Strobe Lights	As Desired.

- Radios & Avionics	Set.

- Autopilot (if installed)	Off.

- Wing Flaps	Set of Takeoff (0-10°).

- Brakes	Release.

Takeoff

- Wing Flaps	0°-10°.

- Throttle	Full Open.

- Mixture	….Rich (above 3000 ft., lean to obtain max. RPM).

- Elevator Control	..Lift Nose Wheel (at 55 KIAS).

- Climb Speed	70 - 80 KIAS.

- Short Field T.O.	10° Flaps / 56 KIAS Until Clear Obstacles.

- Soft Field T.O	10° Flaps / Ground Effect ASAP.

6.  Wing Flaps	Retract at safe altitude above 75 KIAS.

Enroute Climb

- Airspeed	70 - 85 KIAS.

Note

If a maximum performance climb is necessary, use speeds shown in the Max Rate Of Climb chart in section 5 of the POH.

- Throttle	..Full Open.

- Mixture….. Rich (above 3000 ft., lean for max. RPM).

Cruise

- Power	2100-2700 RPM (no more than 75% is recommended).

- Elevator Trim	Adjust.

- Mixture	Lean.

Descent

- Power	As Desired.

- Mixture	Adjust for smooth ops (full rich for idle power).

- Altimeter	Set.

- NAV/GPS Switch (if installed)	Set.

- Fuel Selector Valve	Both.

- Wing Flaps	As Desired.

## Before Landing

- Pilot and Passenger Seat Backs	 Most Upright Position.

- Seats & Seat Belts	..Secured & Lock.

- Fuel Selector Valve	Both.

- Mixture Control	Rich.

- Landing & Taxi Lights	On.

- Autopilot (if installed)	Off.

Normal Landing

- Airspeed	65 - 75 KIAS (flaps up).

- Wing Flaps	As Desired (0º - 10º below 110 KIAS, 10º -30º below 85 KIAS).

- Airspeed	..60 - 70 KIAS (Flaps Down).

- Touchdown	Main Wheel First.

- Landing Roll	Lower Nose Wheel Gently.

- Braking	Minimum Required.

## Balked Landing

- Throttle	Full Open.

- Wing Flaps	Retract to 20°.

- Climb Speed	60 KIAS.

- Wing Flaps	..10º (until obstacles are cleared). RETRACT (after reaching a safe altitude and 65 KIAS)

After Landing (Clear of Runway)

- Wing Flaps	Up.

- Lights	As Required.

- Transponder	GND/STBY & 1200.

- Mixture	Lean.

- Pitot Heat	Off.

# Securing Aircraft

- Parking Brake	Set.

- Electrical Equipment, Autopilot (if installed)	Off.

- Avionics Master Switch	Off.

- Magnetos	Check for Ground.

- Mixture	Idle Cut Off.

- Ignition Switch	Off.

- Master Switch	Off.

- Control/Avionics Lock	Install.

- Parking Brake 	 Off.

- Fuel Selector	Left or Right.

- Aircraft	Secured & Locked.

- Flight Plan	Closed.

# V Speeds and Specs

- X-Wind (Max Demo’d)	15 Knots

- Best Glide Speed	..68 KIAS (flaps up)

- Vx (Sea Level)	62 KIAS

- Vy (Sea Level)	74 KIAS

- Vso  Stall w/ Flaps	40 KIAS

- Vs1  Stall w/o Flaps	48 KIAS

- Va  Max Abrupt Ctrl (2550 Lbs)	105 KIAS

- Va  Max Abrupt Ctrl (2200 Lbs)	98 KIAS

- Va Max Abrupt Ctrl (1900 Lbs)	90 KIAS

- Vno  Max Structural Cruise	..129 KIAS

- Vne  Never Exceed	163 KIAS

V Speeds and Specs are based on sea level. Consult the Air Plains Services, Corp. FAA Approved Airplane Flight Manual Supplement for V speed and Specs for operations above sea level.

# Aircraft Information

- Gross Weight	2550 lbs.

- Engine	Lycoming  IO-360-L2A

- Max Power	180 BHP

- Max Engine Speed	2700 RPM

- Fuel Type	100LL (Blue)

- Fuel Capacity	53 Gal Usable

- Oil Capacity	8 Qts (Minimum 5)

- Electrical	24 - 28 Volt / 60 Amp

- Tire Pressure	Nose-45 PSI / Main-38 PSI

This checklist is a guide to coordinate Pilot Operating Handbook and STC data applicable to this particular aircraft only.  The applicable Pilot Operating Handbook and STC installations remain the official documentation for this aircraft.  The pilot in command is responsible for complying with all items in the Pilot Operating Handbook and applicable STCs. I certify this checklist has been reviewed for accuracy.

Michael J Mueller, 2 February 2010

________________________________________

Wing Director of Maintenance         Date