#### Cessna-182T Nav III – N899CP

# Preflight Cabin

- AIF Front Cover………...Inspect for Airworthiness

- Pitot Tube Cover ..	Remove. Check for blockage.

- POH	Accessible to Pilot.

- Garmin G1000™ Cockpit Reference Guide	Accessible to Pilot.

- Weight & Balance	Checked.

- Parking Brake	Set.

- Control Wheel Lock	Remove.

WARNING

When the master switch is on, using an external power source, or manually rotating the propeller, treat the propeller as if the magnetos switch were on. Do not stand, nor allow anyone else to stand, within the arc of the propeller since a loose or broken wire, or a component malfunction could cause the engine to start.

- MAGNETOS Switch	Off.

- AVN Switch (BUS 1&2)	Off.

- Beacon……………………………On.

- MASTER Switch (ALT &BAT)	On.

- Flaps………………… as Required.

- Primary Flt Display	Verify On.

- Check FUEL QTY (L&R)                                 Hobbs & Tach  Check.

- LOW FUEL L & R Annunciators	Verify Off.

- Fuel Selector…………………Both.                    Reset fuel gauge

- OIL PRESS Annunciator	Verify On.

- LOW VOLTS Annunciator	……On.

- LOW VACUUM Annunciator….On.

- AVIONICS Switch (BUS 1)	On.

- Forward Avionics Fan.	Check Audibly for Operation.

- AVIONICS Switch (BUS 1)	Off.

- AVIONICS Switch (BUS 2)	On.

- Check MFD for correct A\C type and Jeppesen expiration dates, then press ENT

- Aft Avionics Fan……..	Check Audibly for Operation.

- AVIONICS Switch (BUS 2)	Off.

- PITOT HEAT Switch	On/Check.

- Stall Warning System	Check.

- PITOT HEAT Switch	Off.

- MASTER Switch (ALT & BAT)	Off.

- Trim Controls	Takeoff position.

- FUEL SELECTOR Valve	Both.

- ALT STATIC AIR Valve	Off.

- Fire Extinguisher	Verify green.

Preflight Empennage

- Baggage Compartment Door    Unlocked ………..CHECK latched

- Rudder Gust Lock	Remove.

- Tail Tie-Down	Disconnect.

- Control Surfaces	.Check.

- Trim Tab	Check for security.

- Antennas	Check.

Preflight Right Wing trailing edge

- Aileron	Check.

- Flap	Check.

# Preflight Right Wing

- Wing Tie Down	Disconnect.

- Wing Tank Vent Opening	Check.

- Main Wheel Tire	Check…42psi.

- Fuel Tank Quick Drain Valves.	Drain

5 Drains per wing

- Fuel Quantity	Check Visually.

- Fuel Filler Cap	Secure and Vent Unobstructed.

Nose

- Static Source Opening	Check.

- Fuel Strainer Quick Drain	Drain.

3 Belly drains

- Engine Oil Dipstick	Check oil level and secure. (4 qt min., 9 qt for extended flights)      Operate between 7 & 8 qts

- Engine Cooling Outlets	Clear.

- Propeller & Spinner	Check.

- Air Filter	Check.

- Nose wheel Strut and Tire	Check.

- Pressure……………………….49psi.

- Static Source Opening	Check.

# Preflight Left Wing

- Wing Tie-down	Disconnect.

- Left Fuel Quantity	Visually Check.

- Fuel Filler Cap	Secure & Vent unobstructed.

- Fuel Tank Quick Drain Valves.	Drain.

5 Drains per wing

- Main Wheel Tire	Check……42psi.

# Preflight Left Wing Leading Edge

- Fuel Tank Vent Opening	Check for blockage.

- Stall Warning Opening	Check for blockage.

- Landing/Taxi light(s)	Check.

# Preflight Left Wing Trailing Edge

- Left Aileron	Check.

- Left Flap	Check.

##### PASSENGER BRIEF

- Seat Belts / Shoulder Harness

- Personal Electronic Devices off

- Air Vents / Comfort

- Fire Extinguisher Location / Operation

- Emergency Procedures & Exits

##### MISSION BRIEF

- Mission Objective

- Destination, WX, Route, Alt, ETE

- NOTAMS

- Crew Coordination & CRM

- Sterile Cockpit Procedures

- Cockpit Layout

- Intercom & Radio Usage

- Seats, Seatbelts, Doors

- Emergency Action & Equipment

Before Starting Engine

- Preflight Inspection	Complete.

- Passenger Brief	Complete.

- Seats / Belts / Shoulder Harness	Adjust and lock, check initial reel (front & rear).

- Brakes	Test & Set.

- Circuit Breakers	Check In.

- Electrical Equipment	Off.

- Avionics Switch (Bus 1&2)	Off

- Cowl Flaps	Open.

# Starting Engine (Using Battery)

- Beacon Light Switch	….On

- Throttle Control	Open ¼ Inch.

- Propeller Control	High RPM.

- Mixture Control	Idle Cut Off.

- Stby Batt Switch	Test (Hold for 10 seconds, verify that green test lamp does not go out), then ARM (verify that PFD comes on).

- Engine Indicating System	Check parameters, (verify no red X’s through ENGINE page indicators).

- Bus E Volts	Verify 24 volts min.

- M Bus Volts	Verify 0 volts.

- Batt S Amps	Verify Discharge (neg).

- Stby Batt Annunciator	Verify On.

- Propeller Area	Clear.

- Master Switch (Alt and Bat)	On.

Note

If engine is warm, omit priming procedure of steps 13, 14 and 15 below.

- Fuel Pump Switch	On.

- Mixture Control..	Advance to Full Rich, wait until fuel flow indication is stable, then return to idle cut off position.

- Fuel Pump Switch	Off

- Magnetos Switch	  ………..…..Start.

- Mixture Control..	Advance to full rich when engine starts.

Note

If the engine floods, place the mixture control in the Idle Cut Off position, open the throttle control ½ to full, and engage the starter motor (Start).  When the engine starts, advance the mixture control to the Full Rich position and retard the throttle control promptly.

- Oil Pressure	Check.

- Amps (M Batt & S Batt)	Check charge (positive).

- Low Volts Annunciator	Verify Off.

- Nav Lights Switch	On as req.

- Avionics Switch (Bus1&2)	On.

- MFD………………….…Press ENT

- ATIS / AWOS	……………Copy

- Flaps   ……………………….      UP

Taxi

- Mixture Control	Lean as required.

- Brakes	Test.

- Heat / Vents / Defrost	As Required.

- Attitude Indicator	Verify Proper Ops.

- Turn Coordinator	Verify Ops.

- H.I. & Compass	Verify Proper Ops.

- Fuel Selector Valve	Both.

Before Takeoff - Run-Up

- Parking Brake	Set.

- Passenger Seat Backs…Most upright position.

- Seats and Seat Belts..	Check Secure.

- Cabin Doors	Closed and Locked.

- Flight Controls	Free & Correct.

- Flight Instruments..	Check no red Xs.

- Altimeter:

- PFD (Baro)	Set.

- Standby Altimeter	Set.

- KAP 140 Autopilot (Baro)	Set.

- G1000 Alt Sel	Set.

- KAP 140 Altitude Preselect	Set.

Note

There is no connection between the G1000 Alt Sel feature and the KAP 140 autopilot altitude pre-select or altitude hold functions.  G1000 and KAP 140 altitudes are set independently.

- Standby Flight Instruments	Check.

- Fuel Quantity	Check.

Note

Flight is not recommended when both fuel quantity indicators are in the yellow arc range..

- Fuel Selector Valve..	Recheck Both.

- Manual Electric Trim (MET)	Check.

- Elevator & Rudder Trim..	Set for Take Off.

- Mixture control	    Rich    	Throttle Control	1800 RPM.

- Magnetos Switch	Check (RPM drop 175 or 50 differential between magnetos.)

- Prop Control	..Cycle from high to low RPM, return to high RPM (full in).

- VAC Indicators	Check.

- Engine Indicators	Check.

- Ammeters & Voltmeters.	Check.

- Annunciators	Check none illuminated.

- Throttle Control	Check Idle.

- Throttle Control	1000 RPM or less.

- Throttle Friction Lock	Adjust.

- Com Frequency(s)	Set.

- Nav Frequency(s)	Set.

- FMS/GPS Flight Plan	As Desired.

Check GPS 2 avail. on Aux  page,3

- XPDR	Set.

- CDI Softkey	Select NAV source.

Caution (See Full Caution in POH)

The G1000 HSI does not provide a warning “Flags”. The missing D-Bar is considered to be the warning flag.

WARNING

(See Full Warning in POH)

Interruption of NAV signal to the autopilot will cause autopilot to revert to ROL mode with NO warning chime or PFD annunciation.

- Auto Pilot………………………..OFF

- Wing Flaps	0°-20° (10º preferred).

- Cowl Flaps	Open.

- Cabin Windows	Closed & Locked.

- Strobe Lights Switch….as required

- Brakes	Release.

Takeoff

- Flaps	0°-20° (10º preferred).

- Throttle Control	Full.

- Propeller Control	2400 RPM.

- Mixture Control	….Full Rich, above 5000 ft. alt., lean for max. RPM.

- Rotate	50-60 KIAS.

- Normal Climb Speed	80 KIAS.

- Short Field T.O.	20° Flaps / 58 KIAS Until Clear.

- Soft Field T.O	20° Flaps / Ground Effect ASAP.

7.  Wing Flaps	Retract at safe altitude.

Normal Climb

- Airspeed	85-95 KIAS.

- Throttle	..23 Inches or Full (If less than 23 in. Hg.).

- Propeller Control	2400 RPM.

- Mixture…..15 GPH or Full Rich (If less than 15 GPH).

- Fuel Selector	Both.

- Cowl Flaps	Open as required.

Cruise

- Power	15-23 In. & 2000-2400 RPM (no more than 80%).

- Elevator & Rudder Trim	Adjust.

- Mixture	Lean.

- Cowl Flaps	Closed or as required.

- FMS/GPS..	Review & Brief.

Descent

- Power	As Desired.

- Mixture	Enrich as required.

- Cowl Flaps	Closed.

- Altimeters:

- PDF (Baro)	Set.

- Standby Altimeter	Set.

- KAP 140 Autopilot (Baro)	Set.

- G1000 Alt Sel	Set.

- KAP 140 Altitude Preselect	Set.

Note

See note 1 under Before Takeoff – Run-Up.

- CDI Soft key	Select NAV source.

- FMS/GPS 	Review & Brief.

See Caution in Before Takeoff Run-up.

- Fuel Selector valve	Both.

## Wing Flaps	As Desired.

## Before Landing

- Pilot and Passenger Seat Backs	 Most Upright Position.

- Seats & Seat Belts	..Secured & Lock.

- Fuel Selector	Both.

- Mixture Control	Rich.

- Propeller Control	High RPM.

- Landing & Taxi Light Switches	On.

- Autopilot	Off.

Normal Landing

- Airspeed	70-80 KIAS (Flaps Up).

- Wing Flaps	As Desired.

- Airspeed	60-70 KIAS (Full Flaps).

- Trim	Adjust.

- Touchdown	Main Wheel First.

- Landing Roll	Lower Nose wheel.

- Braking	As Required.

## Balked Landing

- Power	Full Throttle & 2400 RPM.

- Wing Flaps	Retract to 20°.

- Climb Speed	55 KIAS.

- Flaps	..Retract Slowly (above 70 KIAS).

- Cowl Flaps	Open.

After Landing (Clear of Runway)

- Wing Flaps	Up.

- Cowl Flaps	Open.

- Lights	As Required.

- Transponder	GND/STBY & 1200.

- Mixture	Lean.

- Pitot Heat	Off.

# Securing Aircraft

- Parking Brake	Set.

- Throttle Control	Idle.

- Electrical Equipment	Off.

- Avionics Switch (Bus 1&2)	Off.

- Mixture	Idle Cut Off.

- Magneto & Master Switch	Off.

- Stby Batt Switch	Off.

- Control Lock	Install.

- Parking Brake 	 Off.

- Cowl Flaps 	Closed.

- Fuel Selector	Left or Right.

- Aircraft	Secured & Locked.

- Flight Plan	Closed.

This checklist is a guide to coordinate Pilot Operating Handbook and STC data applicable to this particular aircraft only.  The applicable Pilot Operating Handbook and STC installations remain the official documentation for this aircraft.

The pilot in command is responsible for complying with all items in the Pilot Operating Handbook and applicable STCs.

I certify this checklist has been reviewed for accuracy.

Michael J Mueller, 2 February 2010

______________________________________

Wisconsin Wing Director of Maintenance

Date